PERMITTEE

Waste Management Inc. of Florida

3750 White Lake Boulevard

Naples, Florida    34117

PERMITTING AUTHORITY

Florida Department of Environmental Protection (Department)

Division of Air Resource Management

Office of Permitting and Compliance

2600 Blair Stone Road, MS #5505

Tallahassee, Florida 32399-2400

PROJECT

Air Permit No. 0210051-027-AC

Minor Air Construction Permit

Collier County Landfill

This project authorizes a like for like replacement of the currently permitted 3,000 standard cubic feet per minute (scfm) open flare (EU003).

NOTICE AND PUBLICATION

The Department distributed a draft minor air construction permit package on October 9, 2020.    The applicant published the Public Notice in the Naples Daily News on October 14, 2020.    The Department received the proof of publication on October 16, 2020.    No requests for administrative hearings or requests for extensions of time to file a petition for administrative hearing were received.

COMMENTS

Public

On October 26, 2020, the Department received comments from Michael R. Ramsey, on behalf of the Golden Gate Estates Area Civic Association.    Mr. Ramsey states that in the last 10 years the residents of the Estates Community has had to deal with 2-3 major wildfires and 1 major open fire issue with an improperly operated vegetative recycling business (Environmental Turnkey Solutions; ETS) in Collier County, and that in all cases there was insufficient attention paid to operational issues that led to these community threatening events.    The following summarizes the comments and the Department’s response.

1. Is the methane gas produced at the landfill a day to day significant ignition/fire source?

Response:    By federal law (Title 40 of the Code of Federal Regulations (CFR), Part 60, Subpart WWW, Standards of Performance for Municipal Solid Waste Landfills, the Collier County Landfill is required to collect the landfill gas (LFG) generated from the decomposition of waste under anerobic conditions in the landfill.    The LFG produced at the landfill primarily consists of 45-50 percent carbon dioxide, 45-50 percent methane, and trace amounts of nitrogen, oxygen, ammonia, sulfides, hydrogen, carbon monoxide, and nonmethane organic compounds (NMOCs).    The collected LFG is passed through a gas treatment system and the sent to the five 0.8-megawatt spark ignition reciprocating internal combustion engines (RICE) to produce electricity.    The remainder of the collected LFG is sent to the three existing open flares – a 3,000 standard cubic feet per minute (scfm) flare and two 1,200 scfm flares.    Since the vast majority of the LFG generated by the landfill is collected and the combusted    under controlled conditions via five RICE engines and the three open flares, the Department does not believe that the methane gas produced at the landfill is a significant ignition/fire source.

2. Are the vehicles, machines and other electrical devices operated onsite considered an “ignition source,” safe not to start an explosion / wildfire event in the community?

Response

:    This permitting action solely deals with the authorization the installation of a replacement flare to combust LFG generated by the landfill.    This conce

rn brought forth by the commenter is not germane to this permitting action.    However, the replaced flare will ensure that along with the other RICE engines and other flares on site, the vast majority of generated LFG will be collected and combusted in a s

a

fe manner..

3. Can embers from an adjacent wildfire cause an explosion or enhance wildfire conditions to the methane gas source onsite?

Response:    Once again, this concern brought forth by the commenter is not germane to this permitting action.    However, as previously stated, The vast majority of the generated LFG is collected and combusted under controlled conditions.

4. Are there periodic checks to detect if methane is leaking from the source area?

Response:    40 CFR 60, subpart WWW, referenced in response to comment 1 requires the landfill to operate the landfill gas collection system so that the methane concentration is less than 500 parts per million above background concentration at the surface of the landfill.    To determine if this level is exceeded, the landfill is required to conduct quarterly surface testing around the perimeter of the collection area and along a pattern that traverses the landfill at 30 meter intervals and where visual observations indicate elevated concentrations of landfill gas, such as distressed vegetation and cracks or seeps in the cover.

5. This location is less than 1000 feet from the Collier County Paradise Sports Park.    Will the methane gas and the flare present a danger to this unaware people from inhalation, fire or explosion?

Response:    Please see responses to Comments 1 and 4.

6. Will smell be an issue to people using the Paradise Sport Park?

Response:    The LFG collection system and associated combustion devices (RICE engines and flares) ensure that most LFG (main odor source for a landfill) is collected and destroyed.    This permitting action, the like-for-like replacement of the 3,000 scfm open flare help in reducing instances of odors.    Despite these efforts to control odors, if the people using Paradise Sport Park experience objectionable odors, please report them to the Department’s South District office by phone at (239) 344-5600 or by email at SouthDistrict@FloridaDEP.gov.

7. What are the emergency response protocols, procedures, neighborhood notifications for methane, fire, explosions?

Response:    Pursuant to Section 403.077 of the Florida Statutes (F.S.), the Department has established a method for regulated entities to submit Public Notices of pollution for reportable releases.    These notices are made available to the public by visiting https://floridadep.gov/pollutionnotice or by subscribing to the department’s e-mail subscription service.    Interested parties may subscribe to this e-mail subscription by visiting https://floridadep.gov/pollutionnotice.    To learn more about the safety protocols and procedures that the Collier County Landfill has in place, the Department encourages members of the Golden Gate Estates Area Civic Association to request a meeting with Waste Management Inc. or Collier County.

8. Waste Management Inc. has never met with the community about safety.

Response:    Golden Gate Estates Area Civic Association may request a meeting with Waste Management Inc. or Collier County to discuss safety concerns that the community members have.

CONCLUSION

The final action of the Department is to issue the permit as drafted.